Nirma University Institute of Technology

Course Code :- 6CS105CC26 Course Name :- Computer Network

Practical - 1

Student Name

Thakor Vinay

Student Roll No.

26MCA061

Faculty In Charge

# Dr. Lata Gohil

Computer Networks           6CS105CC26                                      26MCA061            Thakor Vinay

Identification of Various Network-Related Components

# 1. Connections

A network connector is a hardware fitting attached to the end of a cable that allows it to be plugged into a networking device such as a computer, switch, or hub. Two common connector types are described below.

## 1.1 RJ-45 Connector

RJ-45 (Registered Jack 45) is an 8-pin/8-position connector used to terminate twisted pair network cables. It is the standard connector used for Ethernet (LAN) connections.

- Has 8 pins arranged in a single row

- Used with twisted pair (UTP/STP) cables

- Terminated using the T568A or T568B wiring standard

- Commonly used to connect computers, switches, and routers

Advantages

- Easy to install and replace

- Low cost

- Supports high-speed Ethernet standards

Disadvantages

- Limited cable run distance without a repeater

- Susceptible to electromagnetic interference

Image source https://www.fs.com/blog/introduction-of-the-rj45-interface-6747.html

## 1.2 BNC Connector

BNC (Bayonet Neill-Concelman) is a connector used with co-axial cables. It uses a twist-and-lock (bayonet) mechanism to secure the connection.

- Used mainly with co-axial cable

- Provides a quick connect/disconnect bayonet locking mechanism

- Commonly used in older bus-topology (10Base2) Ethernet networks and CCTV systems

Advantages

- Secure locking connection

- Good shielding against interference

Disadvantages

- Largely obsolete for data networks

- Bulkier than RJ-45

Image source: https://en.wikipedia.org/wiki/BNC_connector

# 2. Cables

Network cables are the physical medium through which data signals travel between connected devices. Different cable types suit different network requirements.

## 2.1 Co-axial Cable

Co-axial cable consists of a central copper conductor, an insulating layer, a metallic shield, and an outer jacket. It was widely used in early Ethernet and cable television networks.

- Single copper core surrounded by insulation and shielding

- Offers good resistance to interference due to shielding

Advantages

- Better noise immunity than unshielded cables

- Supports longer cable runs than UTP

Disadvantages

- Bulky and less flexible

- More expensive and harder to install than twisted pair

Image source: https://www.hltechmw.com/blog_detail/coaxial-cables

## 2.2 Twisted Pair Cable

Twisted pair cable consists of pairs of insulated copper wires twisted together to reduce electromagnetic interference and crosstalk.

- Available in shielded (STP) and unshielded (UTP) forms

- Standard medium for modern LAN wiring

Advantages

- Inexpensive and easy to install

- Flexible and lightweight

Disadvantages

- Shorter maximum distance compared to fiber

- More prone to interference than shielded cables

Image source: https://www.wirecablemakingmachine.com/news/shielded-vs-unshielded-network-cable-which-should-you-use.html

## 2.3 UTP Cable

Unshielded Twisted Pair (UTP) cable is the most common form of twisted pair cabling, used extensively for Ethernet networking.

- No metallic shielding around the wire pairs

- Categorised into standards such as Cat5e, Cat6, Cat6a

Advantages

- Low cost and widely available

- Easy to terminate with RJ-45 connectors

Disadvantages

- More susceptible to electromagnetic interference

Image source: https://www.electronicsmedia.info/2018/01/02/unshielded-twisted-pair-and-shielded-twisted-pair-cable/

## 2.4 Cross & Straight Cable

These refer to two different wiring arrangements used when terminating twisted pair cable with RJ-45 connectors, depending on the devices being connected.

- Straight-through cable: both ends wired identically (T568B–T568B); used to connect dissimilar devices, e.g. PC to switch

- Crossover cable: transmit and receive pairs are swapped (T568A–T568B); used to connect similar devices, e.g. PC to PC or switch to switch

Advantages

- Allows flexible direct connections without extra hardware

Disadvantages

- Manual wiring choice can cause connectivity issues if wrong type is used (largely resolved in modern Auto-MDIX devices)

Image source: https://www.computercablestore.com/straight-through-crossover-and-rollover-wiring

# 3. NIC (Network Interface Card)

A Network Interface Card (NIC) is a hardware component that connects a computer to a network, enabling it to send and receive data. It may be a physical expansion card or an integrated chipset on the motherboard.

- Assigns a unique MAC (Media Access Control) address to the device

- Converts data into electrical, optical, or radio signals for transmission

- Available as wired (Ethernet) or wireless (Wi-Fi) NICs

Advantages

- Enables network connectivity for a device

- Wired NICs offer stable, high-speed connections

Disadvantages

- Wireless NICs can face interference and lower reliability

- Faulty NIC drivers can cause connectivity issues

Image source: https://www.qsfptek.com/qt-news/nic-network-interface-card-definition-types-and-functions.html

# 4. Fiber Optics Cable

Fiber optic cable transmits data as pulses of light through thin strands of glass or plastic fiber, rather than electrical signals over copper.

- Consists of a core, cladding, and protective outer jacket

- Available in Single-mode and Multi-mode variants

- Used for high-speed, long-distance data transmission (backbones, ISPs)

Advantages

- Very high bandwidth and speed

- Immune to electromagnetic interference

- Supports much longer distances than copper

Disadvantages

- Higher cost of cable and installation

- More fragile; requires skilled termination and splicing

Image source: https://www.geeksforgeeks.org/computer-networks/fiber-optics-and-types/

# 5. Switch, Hub, and Router

These are networking devices that connect multiple devices within a network and manage the flow of data between them.

## 5.1 Switch

A network switch connects devices within a LAN and forwards data only to the specific device it is intended for, using MAC addresses.

- Operates mainly at the Data Link Layer (Layer 2)

- Maintains a MAC address table to direct traffic intelligently

Advantages

- Reduces network congestion by directing traffic efficiently

- Supports full-duplex communication

Disadvantages

- More expensive than a hub

- Limited routing capability across different networks

Image source: https://en.wikipedia.org/wiki/Network_switch

## 5.2 Hub

A hub is a basic networking device that connects multiple devices and broadcasts incoming data to all connected ports, regardless of the intended recipient.

- Operates at the Physical Layer (Layer 1)

- Does not filter or direct traffic; simply repeats signals

Advantages

- Simple and inexpensive

- Easy to set up

Disadvantages

- Causes network congestion and collisions

- No traffic management or security

- Largely replaced by switches

Image source: https://www.tutorialspoint.com/computer_fundamentals/what_is_the_hub.htm

## 5.3 Router

A router connects multiple networks together and directs data packets between them based on IP addresses, commonly used to connect a LAN to the Internet.

- Operates at the Network Layer (Layer 3)

- Uses routing tables and protocols to determine the best path for data

- Often provides additional features such as NAT, firewall, and DHCP

Advantages

- Connects different networks, including to the Internet

- Provides security features like firewalling and NAT

Disadvantages

- More complex to configure than switches/hubs

- Higher cost

Image source: https://www.geeksforgeeks.org/computer-networks/introduction-of-a-router/

Computer Networks           6CS105CC26                                      26MCA061            Thakor Vinay

Practice Network-Related Commands

# 1. Ping

Ping checks whether the host is reachable through an IP network and estimates the round-trip time (RTT) of packets sent to it.

•	Basic syntax: ping

•	Sends ICMP Echo Request packets and waits for their Echo Reply

•	Reports the round-trip time for each packet sent

•	Useful to check the connectivity and latency quickly

Output

•   Packets sent, received, and lost (packet loss %)

•   Minimum, maximum, and average round-trip time

•   “Request timed out” if the host is unreachable

# 2. Ipconfig

Ipconfig displays the current TCP/IP configuration of a Windows machine and can refresh its DHCP and DNS settings.

- ipconfig – shows basic IP address, subnet mask, and default gateway

- ipconfig /all – shows detailed configuration including MAC address and DNS servers

- ipconfig /release and /renew – release and renew the DHCP-assigned IP address

- ipconfig /flushdns – clears the local DNS resolver cache

Output

- IPv4/IPv6 address, subnet mask, and default gateway per adapter

- Adapter name, physical (MAC) address, and DHCP status

- Confirmation message after release, renew, or flush operations

# 3. Netstat

Netstat displays active network connections, listening ports, and protocol statistics for the local machine.

- netstat – lists active TCP connections

- netstat -a – shows all connections and listening ports

- netstat -n – displays addresses and ports in numerical form

- netstat -b – shows the executable involved in each connection

Output

- Protocol (TCP/UDP), local address, and foreign address with port numbers

- Connection state, e.g. LISTENING, ESTABLISHED, TIME_WAIT

- Process/program name tied to each connection (with -b)

# 4. Tracert

Tracert (traceroute) traces the path packets take to a destination, showing every intermediate router (hop) along the route.

- Basic syntax: tracert <hostname or IP address>

- Sends packets with increasing TTL values to reveal each hop

- Sends three probes per hop and measures RTT for each

Output

- List of intermediate router IP addresses/hostnames (hops)

- Round-trip time for each of the three probes per hop

- Asterisks (*) / “Request timed out” for hops that do not respond